Upgrading Your Legend Using FlashMaster Software

FlashMaster 1.00.00.02

4/28/00

### A step-by-step guide for upgrading the operational firmware your Epilog Legend.

Introduction:

FlashMaster is a software application that enables the user to download new releases of the operational firmware to the Legend. Users can use FlashMaster to re-program the Legend via either the parallel port or the network connection. The user will be periodically supplied with updates to the Legend when new features become available or when enhancements to existing firmware are released. FlashMaster allows the user to incorporate these features into their own machines at their own convenience.

FlashMaster and Legend upgrades will be supplied either on standard 3.5” diskettes or via email.

Upgrading your Legend for the first time is a three step process:

- Install FlashMaster onto your computer – FlashMaster is the program that allows you to download the upgrades from your computer to your Legend.

- Install new Legend firmware onto your computer – The new Legend firmware is sent to you as a compressed file that needs to be extracted (decompressed) onto your hard drive before you can download it to your Legend.

- Install the new firmware from your computer to your Legend – You will use FlashMaster to download the new firmware to your Legend.

* NOTE:  If you already have FlashMaster installed on your computer you can skip step one.

# 1) Installing Flashmaster onto your computer.

From Diskette:  Use Windows Explorer or the Run command to start the FlashMaster Setup.exe program from the disk labeled FlashMaster.  Move through the install dialogs to completion and then FlashMaster is ready to use.  You can keep FlashMaster on your computer for future use.

Via Email:  Download all of the FlashMaster files to a single 3.5” diskette.  Label and save this diskette for your files.  Install from diskette as per the instructions above.

# 2) Installing new Legend firmware onto your computer

From Diskette: Use Windows Explorer or the Run command to access the Legend firmware upgrade file.  Select the Legendxxx.exe file (where xxx is the release number) that is found on the Legend diskette.  Click on “Unzip” to extract the file. By default, the archive is extracted to the users default temporary directory – normally C:\Windows\Temp.  Once the firmware upgrade file is extracted, continue on to step three to install the new firmware release onto your Legend laser.

Via Email:  Download the firmware upgrade file to a standard 3.5” diskette.  Install from diskette as per the instructions above.

*NOTE - It is extremely important that you Unzip the Legendxxx.exe file before trying to upgrade.  The Unzip process will convert the Legendxxx.exe file into a file with a .hex extension.  Use only the file with the .hex extension to upgrade your Legend.

# 3) Installing new firmware from your computer to your Legend

Prior to starting FlashMaster, the user should ensure that the Legend is turned on  and that it is connected to the computer by either the parallel port or network cable.

FlashMaster is started by selecting the Start/Programs/Epilog/FlashMaster menu item from the taskbar in Windows 98/95/NT/2000. Once started, FlashMaster appears:

Click “Pick File”.

Navigate through the “Open” window to the C:\Windows\Temp folder.  The file that was extracted in step 2 above will be listed in this directory as a vxworks_rom.hex file.

Highlight the appropriate vxworks.hex file then click the “Open” button.

*NOTE - It is extremely important that you choose the proper file with a .hex extension.  Sending the wrong file to your Legend will render the upgrade unsuccessful.

The file will appear in FlashMaster:

If the user is using the LPT1 parallel port, they may just press ‘Go’ at this point. If the user is using another parallel port, they should enter it into the ‘Target’ field. ‘LPT2:’ and ‘LPT3:’, are both examples . The colon at the end of the port name is required.

If the user wishes to program the Legend over the network, the TCP/IP address of the Legend in the ‘Target’ field. Once the target is set, the ‘Go’ button can be pressed.

Sending the update to the Legend over the parallel port takes about 4-5 minutes. The computer and the Legend should not be disturbed during this time. On a Windows 95/98 computer the ‘Progress’ field will properly indicated the progress of sending the file over. On a Windows NT/2000 computer, the progress field will only indicate progress in sending to the spooler, which then in turn, sends to the Legend. When using a network to program the Legend, the progress indicator always shows actual progress, regardless of operating system. Sending over the network takes about 13-15 minutes, depending on network traffic. If the power fails or some other problem disconnects the computer from the Legend during the file sending operation, it can be re-tried, once the problem is corrected. If there is a sending problem, there will be an error indicator on the Legend front panel saying: "Programming Receive Failure, please retry". If this is observed, reboot both the host computer (PC) and the Legend and try again. Once the file has been completely received by the Legend, it will begin the act of re-programming itself with the upgrade version of its firmware. The front panel will say:

## "Programming Machine DO NOT DISTURB"

DO NOT POWER DOWN THE LEGEND DURING THIS TIME. If you lose power, or purposefully power down the Legend during this time, your controller board in the Legend will be ruined, and will have to be replaced. This operation will take about 90 seconds. After programming is complete this message will appear:

## "Programming Complete, please reboot"

You may re-boot at this time and you will then have the new version of the Legend firmware installed in your Legend.  There is a README file supplied with the Legend firmware update in the self-extracting archive. Please read it to get the latest information on Legend features and upgrades.